RESPONSE TO CLARIFICATIONS REQUEST — PARKING

| In response to | PIU — “Clarifications request on methodology and data sources” |
| --- | --- |
| Prepared by / date | Zurab Beradze — 26 June 2026 |
| Scope | Parking-related items only (request Items 1, 5, 8, 9) |
| Sources | Output 8 and Output 10; and yerevan-parking.vercel.app |

# Summary

Parking conclusions rest on two project-collected evidence bases — a desk-based supply inventory (15,897 spaces, 15% field-checked) and a field occupancy survey (six representative areas, hourly licence-plate sweeps 07:00–24:00, 29 May–3 June 2026). No prior model or ticketing data was used. Occupancy, duration, turnover and the displacement ratio are observed; supply totals and the scale of removal are inventory-based; the one figure carrying a real assumption — near-100% re-absorption of displaced demand — is conditional and flagged as such.

# Parking data — obtained and used (Item 9)

| Data type | Status | What it supports |
| --- | --- | --- |
| Occupancy | Observed (field) | Peak occupancy 93%; 69% of zones above 85% |
| Duration of stay | Observed | Short-stay kerb: ~63% of stays ≤1 hour |
| Turnover | Observed | 5–11 vehicles per space per day |
| Licence plate (manual) | Observed | Basis for duration/turnover; |
| ANPR / payment | Not available | Not used. Would help payment-compliance & overnight demand if obtained |
| Off-street | Inventory only | Capacity (7,035 spaces); occupancy not measured |

Note: occupancy is a kerbside (on-street) measurement. Off-street occupancy, overnight-residential demand and payment compliance were not observed.

# What each conclusion rests on (Item 1)

| Conclusion | Basis | Confidence |
| --- | --- | --- |
| Supply 15,897 (8,862 on / 7,035 off) | Inventory + 15% check | High |
| Paid-zone share 11.3%; signage; marking | Inventory | High |
| Removal 5,953 (~85% of kerb) | Inventory × corridor design | High (retained ~1,052 preliminary) |
| Occupancy / duration / turnover | Observed | High (weekday daytime) |
| Displaced demand = 55% of removed | Observed (local) | High |
| Re-absorption approaching 100% | Conditional | Depends on opening gated yards |

No parking conclusion depends on a transport model — the displacement figures are empirical, not modelled.

# Methodology — planned vs delivered (Item 5)

| Component | Delivered as |
| --- | --- |
| Supply survey | Desk inventory (360° video, satellite, Yandex) + 15% field cross-check |
| Occupancy survey | Six representative areas (restored after initial descope), replacing the corridor-wide manual survey |

The change was proportionate — the central-median design removes most kerb parking, so detailed temporal data on a baseline that will not survive has limited value. The methodology-paper revisions and correspondence can be supplied as the decision record.

# Limitations for the risk matrix

| Limitation | How to read it |
| --- | --- |
| Off-street capacity counted gross (92% gated yards) | Re-absorption near 100% is a ceiling, conditional on opening those yards (mitigation in Output 10) |
| Daytime window (07:00–24:00) | Under-counts overnight-residential demand |
| Six areas, not full corridor | Representative sample, not a census |
| Retained ~1,052 from concept design | Preliminary until detailed design; removal of 5,953 is the firm figure |

# Reproducibility (Item 8)

The parking figures are reproducible from project assets: the GeoJSON dataset, the KML layer and https://yerevan-parking.vercel.app/