Conceptual Design and Due Diligence for the Yerevan Sustainable Urban Transport and Electric Mobility

Output 10: Parking Analysis Report

October 15, 2026

Table of Contents

Executive Summary	6

1.	Introduction	7

1.1	Project Background	7

1.2	Parking in the Project Context	7

1.3	Purpose and Scope of This Report	7

1.4	Relationship to the Terms of Reference	8

2	Yerevan Parking System Overview	8

2.1	Institutional and Regulatory Context of Parking Management	8

2.2	Historical Development and Nationalization	9

2.3	Regulatory Framework and Legal Authority	10

2.4	The 2024 Tariff Reform and the Subsequent Legal Proceedings	10

2.5	Escalation of Parking Tariffs	10

2.6	The "Red Line" System and Street Identifiers	11

2.7	Grace Periods and Operational Hours	12

2.8	The Role of the ANPR Cameras	12

2.9	Fine Process	13

2.10	The Digital Payment Ecosystem and Financial Integration	13

3	Parking Surveys	14

3.1	Evolution of the Survey Methodology	14

3.2	Desk-Based Parking Supply Inventory	14

3.3	Field Occupancy Survey	15

4	Parking Impacts of the Conceptual Corridor Designs	15

4.1	Overview of the Conceptual Design Approach	15

4.2	Differentiated Impacts by Land Use Context	16

4.3	Impacts on Specific User Groups	16

4.4	Alignment with Policy Framework	16

5	Displacement and Mitigation Assessment	17

5.1	Primary mitigation – the corridor itself	17

5.2	Assessment of Displacement Potential by Corridor Zone	17

5.3	International Best Practices for Displacement Mitigation	18

5.4	Yerevan-Specific Mitigation Framework	19

5.5	Mitigation Measures for Yerevan	19

5.6	Phasing and Implementation Considerations	22

5.7	Limitations and Residual Risks	23

6	Parking Management Recommendations	23

6.1	Rationale for Benchmarking in the Yerevan Context	23

6.2	Precedents and Yerevan Applicability	23

6.3	Mitigation Measure Packages by Sensitivity Zones	24

7	Conclusions (and Next Steps)	27

7.1	Key Conclusions	27

7.2	Further Studies Required	28

8	Sources	28

PROJECT SYNOPSIS

| Project Name | TA-6559 REG: Implementing the Cities Development Initiative for Asia - Conceptual Design and Due Diligence for the Yerevan Sustainable Urban Transport and Electric Mobility Project (54128-001) |
| --- | --- |
| Country | Republic of Armenia |
| Geographic Area | City of Yerevan |
| Contract Number | S208439 |
| Contracting Authority | Asian Development Bank (ADB) |
| Signed on | 26 August 2025 |
| Commencement of service | 11 September 2025 |
| Duration | 12 calendar months |
| End Date | 7 August 2026 |
| CLient / EA | The Executing Agency (EA) is the Yerevan Municipality (YM) |
| PIU | The Yerevan Urban Development Programs Implementation Unit (PIU) |
| Funding Source | Cities Development Initiative for Asia Trust Fund, administered by ADB, through TA 6559-REG: Implementing the Cities Development Initiative for Asia approved on 8 September 2020. |
| Scope of Work | To develop: (i) bus public transport service Plan for Yerevan; (ii) Conceptual Design of two bus priority corridors; (iii) Upgraded depots to accommodate a fleet of battery electric buses; (iv) Traffic and Parking Management Plan; (v) Communications and Branding; and (vi) due diligence for ADB’s consideration of project financing. |
| Key Deliverables | Inception Report (followed by inception workshop) Output 1: Memo describing selection of affected routes for further analysis Output 2: Data collection methodology memo Output 3: Raw and processed datasets for all collected data (Excel), including maps Output 4: Network diagnostics PPT Output 5: Service Plan PowerPoint and report, including high-quality, graphic-designed maps of the network, each route, and changes to existing routes Output 6: Full conceptual-level designs for the bus priority corridors, bus stations, off-corridor stops, utilities, ITS, points along the off-corridor routes, and costings Output 7: 3-D renderings of stations, with three different top structure architectural designs Output 8: Traffic and parking surveys and analysis report Output 9: Traffic Management Plan, including microsimulation Output 10: Parking analysis report Output 11: Report reviewing existing conditions at depots and future plans Output 12: Conceptual design for both depots Output 13: PPTs, handouts, and minutes from three public workshops Output 14: Communication Strategy Output 15: System brand including name and logo Output 16: Stakeholder engagement plan Output 17: IEE/EIA Output 18: LARP Output 19: Financial Management Assessment Report Output 20: Financial Analysis Report and Linked Document Output 21: Economic Analysis Report and Linked Document Output 22: Strategic Procurement Plan Output 23: Terms of reference Output 24: Climate Risk and Adaptation Assessment Output 25: Climate Change Assessment Output 26: Poverty, Social and Gender Assessment Report Output 27: Social Development and Gender Action Plan Output 28: Institutional Assessment Output 29: Institutional and Capacity Building Roadmap Output 30: Report on Knowledge Transfer Activities. |
| Consultant / Contractor | A+S Consult GmbH, Logit Consulting Inc, Exponent Engineers (Pvt.) Limited & POGHOS SHAHINYAN HAYKAZI |
| Contact | Team Leader: Wagner Colombini Martins, wagner.colombini@logiteng.com Focal point: Lívia Delgado Rodrigues, livia.delgado@logiteng.com |

# Executive Summary

This report assesses how the bus priority corridors on Corridors 1 and 2 affect parking in Yerevan and sets out how the displaced demand should be managed. It complements the Traffic and Parking Surveys and Analysis Report (Output 8), which holds the full survey evidence.

Yerevan's corridor kerb is mostly free and weakly organised. Of the 6,095 on-street spaces on the corridors, 86% are free, only 20% carry signage and 14% marking; the paid system run by Parking City Service covers 780 of them in Zones A and B. The 2024 tariff reform raised charges sharply and is still being challenged in the courts.

The v2 conceptual design removes 4,843 of these spaces and retains or re-establishes 1,252 (901 on Corridor 1, 351 on Corridor 2). Within the corridors' 100-metre impact area the removal is 62% of on-street parking and 33% of all parking once off-street spaces are included.

A licence-plate survey of six representative areas shows a kerb that is busy in places but not full across whole areas. At their busiest hour the six areas run at 36% of capacity, from 18% in Malatia-Sebastia to 57% in Kentron and 68% at Gai Avenue. The pressure sits on particular streets: 32 of Kentron's 53 survey zones reach the 85% occupancy target at their busiest hour. Stays are short - 76% last an hour or less - and each space turns over 3.5 to 7.5 times a day. In the surveyed areas 673 vehicles were parked at the busiest hour in the 1,834 spaces to be removed; surviving nearby kerb space can take 58% of them, and the rest depends on off-street capacity that is largely closed residential courtyards today.

The corridors themselves are the primary mitigation: faster and more reliable buses reduce the number of cars that need to park. Around them the report recommends a layered package, scaled to three sensitivity zones: extending the paid red/blue-line system onto side streets within 100 metres of the corridors; charged resident permits limited to one per household; delineating and marking the free kerb that remains; adjusting charges towards the 85% occupancy target on the streets that reach it; and withdrawing the flat annual permit in favour of hourly pricing. Visitor caps are held as a targeted contingency.

The measures should be in place before or together with the removal of corridor parking.

# Introduction

## Project Background

The Yerevan Sustainable Urban Transport and Electric Mobility Project is being prepared with support from the Asian Development Bank to improve the efficiency, safety, and sustainability of urban mobility in Yerevan, Armenia. The project focuses on enhancing public transport performance through the introduction of bus priority corridors, upgrading supporting infrastructure, and implementing complementary traffic and parking management measures.

The conceptual designs under preparation involve the reallocation of street space along the two priority corridors, including the introduction of dedicated bus lanes, improved pedestrian and non-motorised transport facilities, upgraded intersections, and associated public-realm improvements. These interventions are expected to deliver significant benefits in terms of public transport reliability, road safety, and overall corridor functionality, while requiring careful management of impacts on general traffic and parking.

## Parking in the Project Context

On-street parking plays a significant role in shaping travel behaviour, curbside operations, and street performance in Yerevan. Along many arterial and mixed-use streets, parking occupies a substantial share of available curb space and often competes directly with bus operations, pedestrian movement, loading activities, and traffic flow. In several locations, informal or illegal parking practices further reduce effective road capacity and undermine safety and accessibility.

The introduction of bus priority infrastructure inevitably affects existing parking supply, both directly-through the removal or conversion of parking spaces-and indirectly, by changing traffic circulation patterns and curbside functions. If not properly assessed and managed, parking impacts can generate localised pressure on adjacent streets, affect access to businesses and residences, and create implementation risks for corridor projects.

For this reason, parking analysis is treated as a critical enabling component of the overall corridor concept. The objective is not to preserve parking at all costs, but to ensure that necessary reductions in parking supply are understood, justified, and mitigated to the extent reasonably possible, while supporting the project’s broader mobility and safety objectives.

## Purpose and Scope of This Report

This Parking Analysis Report has been prepared as a standalone deliverable under Task 3: Traffic and Parking Analysis of the project Terms of Reference. It forms Output 10 of the assignment and complements the Traffic and Parking Surveys and Analysis Report (Output 8) and the Traffic Management Plan (Output 9).

The purpose of this report is to:

- Provide a clear overview of the existing parking system context along the priority corridors and in Yerevan as a whole;

- Summarise, at a strategic level, the key findings of parking surveys, without duplicating detailed survey documentation;

- Assess the impacts of the conceptual corridor designs on parking supply;

- Evaluate the potential for parking displacement within the defined influence area;

- Identify mitigation options and parking management measures that support corridor implementation; and

- Present evidence-based conclusions on the feasibility and risks associated with parking changes.

This report focuses on parking conditions in Yerevan urban area. Detailed survey methodologies, raw datasets, block-level inventories, and technical maps are documented separately and may be referenced here where relevant.

Revision note: a targeted field occupancy survey has been completed on six representative areas, on ordinary working days. Its measured results - occupancy, duration, and turnover, now underpin the impact and mitigation analysis in this report and are documented in full in the companion Traffic and Parking Surveys and Analysis Report (Output 8).

## Relationship to the Terms of Reference

The content and structure of this report are aligned with the requirements set out in the project Terms of Reference, which call for:

- Documentation of on-street and off-street parking supply;

- Evaluation of parking loss resulting from corridor redesign;

- Analysis of parking displacement and mitigation options; and

- Preparation of a parking analysis report to support decision-making.

By focusing on interpretation, impacts, and management implications rather than survey execution, this report is intended to support informed discussion with the Yerevan Municipality and other stakeholders on how parking can be managed in a manner consistent with the project’s transport, safety, and urban-realm objectives.

# Yerevan Parking System Overview

The urban landscape of Yerevan, the capital of Armenia, has encountered a transformative decade in terms of its transport infrastructure and public space management. As the city has grown in both population and vehicle density, the management of on-street parking has transitioned from an informal, often chaotic arrangement into a highly regulated, technology-driven municipal service. This report serves as a foundational analysis of the current parking ecosystem, evaluating the institutional framework, the socio-economic implications of recent fiscal reforms, and the technical mechanisms of enforcement and data-driven management. By examining the evolution of "Parking City Service" CJSC and the broader regulatory environment, this analysis identifies the critical pressure points in Yerevan’s urban mobility strategy and provides a robust framework for understanding the city's transition toward a "smart city" model of metropolitan governance.

## Institutional and Regulatory Context of Parking Management

Figure 1 - „Parking City Service“ CJSC / www.pcs.am

The governance of parking in Yerevan is anchored in a centralized municipal structure that has undergone significant shifts in ownership and operational philosophy. At the center of this structure is the "Parking City Service" Closed Joint-Stock Company (CJSC), which operates as a specialized utility under the jurisdiction of the Yerevan Municipality. The establishment of this entity was a direct response to the increasing congestion in the city’s administrative districts, particularly the Kentron (Center) and Arabkir areas, where the competition for limited street space had begun to impede the flow of public transit and emergency services.

## Historical Development and Nationalization

The operational history of "Parking City Service" is characterized by its transition from a private enterprise to a nationalized municipal asset. Originally launched in 2013 as a private company, the service was tasked with introducing the "red line" system-a series of designated paid parking zones identified by red pavement markings. The initial private management model was designed to leverage private capital for the installation of video monitoring equipment and the development of a digital payment infrastructure. However, the private operation of public space quickly became a focal point of political and social tension. Questions emerged publicly regarding the distribution of parking revenue between the private operator and the municipality, as a significant portion of the fees and fines collected were retained by private operators rather than being reinvested into the city's general budget.

In late 2016, following a municipal policy decision to bring parking management into direct public administration, the Yerevan Municipality initiated the nationalization of "Parking City Service". The company's ownership was transferred to the city, allowing for a more transparent integration of parking revenue into the municipal transport budget. This institutional shift was not merely a change in ownership but a fundamental change in mission; the service moved from a profit-driven model to a public utility model where parking regulation is viewed as a tool for urban mobility management rather than a simple revenue stream.

Figure 2 - Zone A paid-parking signage and roadside information board, central Yerevan

Figure 3 - Information Boards for Zone A - Red Line, Zone B - Blue Line

## Regulatory Framework and Legal Authority

The legal basis for parking regulation in Yerevan is derived from the Armenian law "On Local Parking Fees for Motor Vehicles". This legislation empowers the Yerevan Council of Elders-the city's primary legislative body-to establish parking zones, set fee structures, and define the parameters of enforcement. The Council of Elders acts as the governing board for municipal policy, frequently debating the balance between fiscal necessity and public affordability.

Under this framework, "Parking City Service" CJSC is responsible for the technical implementation of these policies. Its mandate includes the maintenance of road markings, the operation of the automated Automatic Number Plate Recognition (ANPR) camera network, and the management of the digital back-office that processes parking sessions.

The Director of the CJSC, Van Tadevosyan, oversees these operational functions, ensuring that the service aligns with the broader goals of the Municipality’s Transport Department. On February 24, 2026 an private interview with Mr. Tadevosyan at his workplace was conducted. Parts of this report are based on his responses throughout the aforementioned meeting.

| Regulatory Entity | Primary Responsibility | Legal Basis |
| --- | --- | --- |
| Yerevan Council of Elders | Legislative oversight, fee setting, and zone designation. | RA Law "On Local Parking Fees for Motor Vehicles". |
| Yerevan Municipality Transport Dept. | Strategic coordination with public transit and road police. | Municipal Charter. |
| "Parking City Service" CJSC | Operational execution, ANPR enforcement, and pavement marking. | CJSC Corporate Charter and Municipal Contracts. |
| RA Road Police | Administrative adjudication of traffic and parking violations. | RA Administrative Code. |

This institutional arrangement ensures that parking policy is integrated into the city's wider economic and transport goals. For example, the decision to exempt electric vehicles from parking fees was not merely a parking policy but a strategic move by the Council of Elders to promote green mobility within the capital.

## The 2024 Tariff Reform and the Subsequent Legal Proceedings

One of the most significant developments in the recent history of Yerevan's parking management is the comprehensive tariff reform implemented on January 1, 2024. This reform represented a dramatic departure from the previous uniform pricing model, introducing a highly stratified system of zones and fees designed to use economic levers to manage congestion in the city center.

## Escalation of Parking Tariffs

Prior to 2024, the annual fee for parking in Yerevan's "red zones" was a flat rate of 12,000 AMD, regardless of location or usage frequency. This low cost resulted in consistently high occupancy in the city center, as commuters found it more economical to park on the street than to utilize public transit or off-street facilities. The 2024 reform introduced a two-tiered system-Zone A and Zone B-with pricing that reflects the varying demand for space in different administrative districts.

In Zone A, which covers the most congested parts of the Kentron district, the annual fee was increased from 12,000 AMD to 160,000 AMD, representing an escalation of over 1,200%. Zone B, covering less intensive areas, saw a more moderate increase to 24,000 AMD per year. The hourly rates were also adjusted to 300 AMD in Zone A and 200 AMD in Zone B, creating a pricing structure that encourages short-term turnover over long-term occupancy.

| Duration of Stay | Zone A Tariff (2024-2026) | Zone B Tariff (2024-2026) |
| --- | --- | --- |
| 1 Hour | 300 AMD | 200 AMD |
| 1 Day | 2,000 AMD | 1,000 AMD |
| 1 Week | 5,000 AMD | 2,000 AMD |
| 1 Month | 18,000 AMD | 4,000 AMD |
| 1 Year | 160,000 AMD | 24,000 AMD |

The rationale provided by the municipality for these increases was twofold: to generate revenue for the modernization of the public transport fleet and to "unload" the streets of the city center by discouraging unnecessary private vehicle travel. By increasing the cost of parking to a level that reflects the market value of urban space, the city aimed to force a modal shift toward buses, trolleybuses, and the metro.

## The "Red Line" System and Street Identifiers

The effectiveness of Yerevan's parking system is physically defined by the red and blue line markers and the associated signage that governs the city’s streets. The spatial distribution of these zones reflects a strategic effort to manage high-demand corridors while maintaining accessibility for residents and local businesses.

The lines red, blue and yellow (indicating that parking is prohibited in the area) are the primary visual cue for motorists in Yerevan. These markings indicate that a specific segment of the street is subject to local parking fees during operational hours. The maintenance of these lines is a continuous operational task for "Parking City Service," as weather conditions and heavy traffic flow frequently obscure the markings, leading to potential disputes over the legality of citations.

Complementing the pavement markings are standardized road signs and information boards. These boards are critical for informing drivers about the zone classification (A or B) and the available payment methods, including the specific short-codes for SMS billing.

Figure 4 - Parking map available on www.pcs.am website

## Grace Periods and Operational Hours

A vital feature of the Yerevan parking system is the 15-minute grace period. This period is designed to facilitate quick commercial activity-such as dropping off passengers, loading goods, or making short deliveries-without requiring the driver to engage in the payment process. This allowance is particularly important for the city's small commercial operators and delivery services, which rely on quick access to storefronts. Although, the role of the 15-minute grace period may warrant periodic review as the system matures. It’s typical for cities to offer public-friendly features as this one when introducing new parking systems in the beginning, but when the reform and system mature, it could be argued that the grace period does not serve the system.

Standard operational hours for the paid parking system are from 09:00 to 22:00. Outside of these hours, parking is free of charge. This policy recognizes that the demand for street space changes significantly at night, and the primary goal of the parking fee-congestion management-is less relevant during the late-night and early-morning hours. However, motorists must ensure their vehicles are either removed or paid for by 09:00 to avoid automatic citations generated by the cameras.

## The Role of the ANPR Cameras

"Parking City Service" operates ANPR cameras mounted on poles along the street. These cameras surveil the designated parking zones, capturing the licence plates of every vehicle parked. Enforcement runs entirely through this fixed camera network: a parked vehicle is detected by the camera covering its zone, its plate is matched against the payment record held in the back-office, and a citation is issued from there if no valid session is found. That chain - fixed camera, back-office match, citation - is the complete mechanism by which a parking violation is established in Yerevan. The camera system accounts for the grace feature, a violation is only finalized after the 15-minute period.

## Fine Process

Once a violation is confirmed by the "Parking City Service" back-office, the data is shared to the driver via SMS, post, banking app. The standard penalty for failing to pay the local parking fee is 5,000 AMD.

Camera captures violation → information processed → notification sent via multiple channels (SMS, post, banking app) → fine goes to State Treasury. If the driver still does not pay, the case goes to court to find the owner. The system tracks parking duration through camera timestamps (entry and exit captured). Cameras are linked to police database for vehicle owner identification. Foreign/tourist vehicles without registration may escape fines. When a fine is sent, a video of the violation is included as evidence. The car owner (not driver) is liable.

If the fine is not paid and the vehicle becomes subject to towing, the process is handled by the Police, not the “Parking City Service”.

## The Digital Payment Ecosystem and Financial Integration

The modernization of Yerevan's parking system is heavily dependent on its digital payment infrastructure. The city has prioritized a "cashless" model, integrating parking fees into the broader financial technology landscape of Armenia.

SMS remains one of the most popular methods for short-term parking payment in Yerevan. By texting their license plate number to the short-codes 1045 (Zone A) or 5045 (Zone B), drivers can initiate an hourly parking session. The cost is deducted directly from their mobile balance or added to their monthly bill.

For those who do not wish to use SMS, or for those paying for daily, monthly, or annual subscriptions, a vast network of payment terminals and mobile applications is available.

- Payment Terminals: Devices operated by Telcell, EasyPay, and Idram are ubiquitous in Yerevan, found in most grocery stores, bus stops, and public buildings. These terminals allow for cash or card payments directly into the "Parking City Service" account.

- Mobile Apps: Applications like iDram and Mobidram allow users to manage their parking sessions from their smartphones, providing the ability to extend sessions remotely and track payment history.

- Online Portals: The Yerevan Municipality website (yerevan.am) offers an electronic service portal where residents can pay their annual fees and check for any outstanding parking debts using their vehicle registration data.

This multi-channel payment ecosystem is designed to minimize the "barrier to entry" for legal parking. By providing numerous ways to pay, the municipality reduces the validity of the excuse that a driver was "unable" to make a payment before the camera recorded the vehicle.

One of these subscription options is a flat annual zonal-parking permit, bought once for unlimited use of a parking zone for the year. Because a flat annual fee removes the marginal cost of each additional hour parked, it works against the kerb turnover the corridor strategy depends on. This report recommends withdrawing it in favour of hourly pricing.

During our conversation with the head of PCS, the near future plans of rolling out a mobile app for parking has been revealed. Yerevan will soon have a parking app where drivers will be able to see the parking places on a map, check for available spaces and pay for the parking via the app.

# Parking Surveys

## Evolution of the Survey Methodology

The original requirements set out in the Project Terms of Reference (ToR) called for a full-scale, corridor-wide field survey to document parking supply and occupancy. However, following multiple rounds of technical discussions, iterative revisions of the methodology paper, and the receipt of successive financial proposals from local survey companies, it became clear that such a comprehensive field operation was not feasible within the available resources and project timeframe.

In response, the project team, in close coordination with local experts and the Project Implementation Unit (PIU), refined the approach to prioritize a desk-based parking supply inventory.

## Desk-Based Parking Supply Inventory

The primary component of our data collection is a comprehensive supply inventory developed through a systematic "desk job". This inventory utilizes 360-degree street videos collected during project field missions, supplemented by recent street-level imagery and existing municipal maps.

The objective of this inventory is to map and geo-reference every on-street parking space, including:

- Physical Geometry: Parking orientations (parallel, 45-degree, or perpendicular).

- Regulatory Status: Documenting "red-line", "blue-line", and free parking zones.

- Off-Street Context: Cataloging major off-street facilities on the priority corridors to evaluate their potential to absorb displaced demand.

Results of the comprehensive parking supply inventory survey can be viewed at https://yerevan-parking.vercel.app/

The parking assessment for Yerevan was based on a desk-based parking supply inventory covering the bus priority corridors with a 100-metre influence area. Across Corridors 1 and 2 the survey identified 14,587 parking spaces, including 6,095 on-street spaces on the corridors themselves, 1,730 on-street spaces on adjacent cross-streets, and 6,762 off-street spaces in yards and lots. This broader survey frame matters because the project is concerned not only with the parking currently on the corridors but also with the nearby streets and off-street spaces that may absorb part of the impact once the corridor designs are implemented.

The survey results show that the existing parking system along the corridors is dominated by unregulated supply. Of the on-corridor spaces, 5,248 (86.1%) are free, while only a relatively small proportion fall within Zone A (246 spaces), Zone B (534 spaces) or designated taxi spaces (67). The physical organisation of parking is also weak: only 20% of corridor parking has signage, only 14% has markings, and about 76% is parallel parking. These findings indicate that a large share of the existing parking supply is poorly structured and not managed in a systematic way.

Overall, the survey confirms three key points. First, the project corridors currently contain a large amount of parking supply, much of it free and weakly managed. Second, the planned bus priority design will significantly reduce this supply on the corridor itself. Third, the surrounding street network may provide a basis for mitigation, meaning that the task is not simply to count removed spaces, but to understand how demand can be redistributed and better managed within the wider corridor area. This is the main analytical value of the survey.

## Field Occupancy Survey

While supply is relatively static, occupancy is highly dynamic.

| Metric | Definition | Importance for Yerevan |
| --- | --- | --- |
| Occupancy Rate | The percentage of theoretical capacity currently in use. | Identifies zones where demand exceeds the "85% rule" of efficiency. |
| Duration of Stay | The average time a unique plate is detected in a single location. | Distinguishes between "commuter" parking and "retail" turnover. |
| Turnover Rate | The number of different vehicles utilizing a single space over a 24-hour period. | Measures the economic vitality of commercial streets. |
| Violation Rate | The ratio of undetected/unpaid plates to total detected plates. | Evaluates the enforcement effectiveness and the level of public compliance. |

The target occupancy for on-street parking is generally considered to be 85%. At this level, the street is being utilized effectively, but there is still a 15% vacancy rate-meaning one in every seven spaces is empty. This ensures that an arriving driver can find a spot without "cruising," which is a major contributor to urban congestion and emissions.

A field occupancy survey was carried out on six representative areas (Kentron, Komitas, Gai Avenue, Garegin Nzhdeh, Shiraz/Hasratyan and Malatia-Sebastia) rather than the full corridor, on four separate ordinary working days between 29 May and 3 June 2026. Enumerators made hourly licence-plate sweeps from 07:00 to 24:00, recording for every parked vehicle the hour, the zone, the plate, the vehicle type, the kerb location, the parking method and the legality - 21,186 on-street sightings of 10,135 distinct vehicles across 203 survey zones - short segments of street, typically one block face, each checked on every hourly round - in the six areas (Kentron 53; Komitas 53; Shiraz/Hasratyan 34; Garegin Nzhdeh 31; Malatia-Sebastia 22; Gai Avenue 10), plus one off-street facility per area. Occupancy counts each vehicle for half an hour in the hours it arrives and leaves and for a full hour in between, since the hourly rounds record which vehicles were present but not for how long. The measured results are summarised in this report; the full methodology, the survey locations and the results broken down by area are set out in the field occupancy survey chapter of the companion Traffic and Parking Surveys and Analysis Report (Output 8).

The survey was designed to establish an evidence-based baseline for kerbside use to inform the design of the bus priority corridors, capturing both formal red-line areas and free parking practice. The Parking Survey Methodology (v. 18052026) set out three defining features, all of which were executed: a 17-hour observation window (07:00 to 24:00) covering the full daily cycle of commuter and residential activity; manual licence-plate recording, which is what makes turnover and duration measurable rather than estimated; and systematic logging of parking that encroaches on footways, bus stops and second lanes. The one element that could not be delivered was payment compliance, which is observable only in the Parking City Service back-office.

# Parking Impacts of the Conceptual Corridor Designs

## Overview of the Conceptual Design Approach

The conceptual corridor designs developed under Task 2 of the project represent a fundamental reallocation of street space.

The cross-sections developed by the design team allocate the available carriageway width to dedicated bus lanes (2 x 3.5 m), mixed-traffic lanes (2 x 3.0 m per direction), median platforms and separators (0.5 m buffers), and minimum sidewalk widths of 2.50 m on each side. In station areas, additional width is required to accommodate 18-metre platform structures, turnstiles and bike parking facilities. Where the existing right-of-way is fully consumed by that allocation, no kerbside parking can be retained, and this is the case along most of the alignment. Between stations, however, the cross-section is less demanding and in most such sections there is room for parking to remain. That is why the design retains or re-establishes 1,252 spaces (21% of the on-corridor supply) rather than removing all of it. These retained-space figures - 1,252 in total, 901 on Corridor 1 and 351 on Corridor 2 - are taken from the v2 conceptual design and are indicative. The corridor design is still being developed, and each iteration changes where parking can be kept; figures reported elsewhere in the project, including in Output 6, may differ where they reflect a different design iteration.

## Differentiated Impacts by Land Use Context

The impact of parking removal varies significantly depending on the land-use context of each corridor segment. In the Kentron (Centre) district, where sections of Corridor 1 pass through Zone A of the current parking tariff system, on-street parking serves a mix of short-term commercial visitors, delivery and loading operations, and long-stay commuter parking. The 2024 tariff reform, with its 300 AMD hourly rate for Zone A, was specifically designed to encourage turnover and discourage all-day occupancy. The removal of parking from these central segments will disproportionately affect short-term visitors and loading operations, while the displacement of commuter parking is arguably aligned with the project objectives of modal shift toward public transport.

In residential segments of Corridor 2, particularly through the Nor Nork and Malatia-Sebastia districts, the parking that will be removed is predominantly used for overnight residential storage. It should be said plainly that these areas are not without parking. Aerial imagery and the field survey both show substantial parking between the blocks: along the internal streets, in the sheds and lock-up garages that occupy parts of the courtyards. Malatia-Sebastia in fact records the lowest daytime kerb pressure of the six surveyed areas (18% at its busiest hour) and the highest share of parking standing off the carriageway (30.9%).

Along Arshakunyats Avenue, the wide right-of-way (typically exceeding 30 metres) has historically accommodated multiple lanes of traffic alongside parking. The conceptual design uses this width to provide a comprehensive cross-section including bus lanes, two mixed-traffic lanes per direction, cycle lanes and generous sidewalks. The width is largely consumed by that allocation, but not entirely: Arshakunyats retains parking in the sections where the right-of-way allows, and Corridor 1 as a whole retains 901 of its 2,349 on-corridor spaces.

The field survey confirms and quantifies these differences. The Kentron commercial core runs at 57% occupancy at its busiest hour, but 32 of its 53 zones reach the 85% target and 18 read above 100% at their own peak, and it is short-stay dominated (about 67% of vehicles stay one hour or less). The Malatia-Sebastia residential segment shows the lowest daytime pressure (18% at its busiest hour) and a marked long-stay / overnight share (8% of stays exceed eight hours), consistent with residential storage rather than turnover. Turnover across the surveyed areas runs at 3.5 to 7.5 vehicles per space per day.

## Impacts on Specific User Groups

The removal of corridor parking affects several distinct user groups, each with different levels of sensitivity and adaptability. Residents along the corridors, particularly in areas without private garages or courtyards, face the most direct impact. Their parking needs are typically for long-duration overnight storage and are least easily served by alternative locations. Commercial businesses that depend on customer access by car, such as retail shops, restaurants, and medical offices along the central sections, may experience reduced footfall if customers perceive parking as unavailable, although European experience with bus corridor implementation suggests that improved public transport access and enhanced streetscape quality ultimately benefit commercial activity.

Persons with disabilities who depend on proximate parking access are a particularly vulnerable group. Armenian law and international accessibility standards require that accessible parking be maintained within reasonable proximity to key destinations. The corridor designs must ensure that accessible parking provision is specifically addressed, either through retained on-street spaces at designated locations or through clearly marked accessible parking on immediately adjacent side streets.

## Alignment with Policy Framework

The removal of on-street parking from the bus priority corridors is fully consistent with the strategic policy direction established for Yerevan. The Yerevan Sustainable Urban Transport Strategy adopts a modal hierarchy in which sustainable transport modes take priority, and explicitly states that road space on strategic streets will be prioritised for moving people in sustainable transport and not for parking cars. The 2022 Mott MacDonald Parking and Taxi Management Strategy similarly identifies that the roll-out of new priority bus infrastructure will see the removal of some existing parking spaces on certain streets, and recommends that parking provision be withdrawn from streets along which strategic bus services operate.

At the national level, the Government of Armenia, through its collaboration with the Asian Development Bank, has signalled a clear commitment to sustainable urban transport. The Terms of Reference for the current project explicitly anticipate that the development of bus priority corridors may impact parking supply and require the Consultant to evaluate options for displaced parking. The parking removal is therefore not an unintended consequence but a planned and necessary element of the corridor transformation.

# Displacement and Mitigation Assessment

## Primary mitigation – the corridor itself

Parking displacement refers to the relocation of parking demand from the corridor to adjacent streets, off-street facilities, or alternative modes of access when on-street supply is removed. The degree to which displacement occurs, and whether it creates problems, depends on several factors: the volume of parking removed relative to surrounding capacity, the availability and proximity of alternative parking locations, the willingness and ability of users to adjust their behaviour, and the management measures put in place to absorb and redirect demand.

International experience with bus priority corridors in European cities demonstrates that the actual level of displaced parking that needs to be physically accommodated is typically lower than the number of spaces removed. This is because a significant proportion of users shift to other modes (particularly the improved public transport that the corridor provides), adjust their trip timing or destination, or were previously engaged in illegal or informal parking that should not be replicated elsewhere. The European BHLS (Buses with High Level of Service) experience, documented through projects in Nantes, Metz, Rouen, Eindhoven, Helsingborg, and other cities, consistently shows that well-implemented bus priority corridors generate net positive outcomes for corridor liveability and commercial viability, even where substantial parking removal was required.

In other words, the bus priority corridors are themselves the primary mitigation: by delivering faster, more reliable public transport they reduce the mode share of private cars, and so the number of vehicles needing to park. Some users shift modes, some adjust trip timing, and some no longer park on the corridor at all. Every other measure in the chapters that follow complements that structural shift.

## Assessment of Displacement Potential by Corridor Zone

The displacement assessment considers three categories of corridor segments, differentiated by the severity of the parking pressure and the availability of mitigation options.

High-sensitivity zones include the central sections of Corridor 1 through the Kentron district, where parking demand is intense and the side streets are themselves under significant parking pressure, together with Komitas Avenue. In these areas the potential for simple displacement to adjacent streets is limited and more comprehensive management interventions are required. The Amiryan, Nalbandyan and Tigran Mets segments fall into this category, as does Komitas Avenue, which the field survey moved here from the medium band.

Medium-sensitivity zones encompass corridor segments where parking demand is moderate and some absorptive capacity exists on parallel or perpendicular streets within 100 metres. The mid-sections of Arshakunyats Avenue, the Kievyan segment and Garegin Nzhdeh fall into this category. Here, managed displacement to side streets, combined with improved delineation and enforcement, can accommodate a meaningful share of the removed parking demand.

Lower-sensitivity zones include the outer sections of both corridors. In these areas, the surrounding street network typically has greater spare capacity, lower existing occupancy, and a more suburban character that permits more flexible parking accommodation. Displacement in these zones is less likely to generate significant conflict.

Measured recalibration (field survey). The survey tests these a-priori labels against behaviour. Across the surveyed areas the design removes 1,834 spaces, in which an average of 673 vehicles were parked at each area's busiest hour, so measured displacement falls well below the supply removed. Beneath that aggregate the areas differ. Kentron is confirmed high-sensitivity: 57% at its busiest hour overall, but 32 of its 53 zones reach the 85% target and only 16% of its displaced demand can be absorbed on nearby kerb. Komitas, previously medium, is reclassified as high-sensitivity throughout this report: at 46% at its busiest hour it is less full than Kentron, but none of its displaced demand can be absorbed on nearby kerb. Shiraz/Hasratyan and Malatia-Sebastia behave as genuinely lower-sensitivity, at 23% and 18% at their busiest hours. Gai Avenue, at 68%, is the fullest of the six.

## International Best Practices for Displacement Mitigation

European cities that have implemented bus priority corridors and BHLS systems have developed a range of proven strategies for managing parking displacement. These strategies are not applied in isolation but as integrated packages tailored to local conditions. The following synthesis draws primarily on European experience, supplemented by relevant examples from other regions where applicable to the Yerevan context.

Controlled Parking Zones (CPZ) on adjacent streets represent the single most important mitigation tool available. When on-street parking is removed from a bus corridor, the demand pressure shifts to neighbouring streets. Without management, this leads to uncontrolled overflow parking that creates conflict with residents and degrades the local environment. The introduction of zone-based residents parking permits, time-limited visitor parking, and appropriate enforcement on streets within the corridor influence area (typically 100-200 metres from the corridor) ensures that displaced demand is absorbed in an orderly manner. Cities such as London, Paris, Amsterdam, Barcelona, Bologna, and Tbilisi have all adopted zoned parking controls as a core element of their urban transport management, with the 2022 Mott MacDonald Parking Strategy for Yerevan recommending a similar approach.

Conversion of informal to formal parking on side streets is a particularly relevant measure for Yerevan, where significant numbers of vehicles park informally or illegally on sidewalks, in unmarked areas, and in configurations (perpendicular and echelon) that use street space inefficiently. By surveying side streets within the 100-metre influence area and formally delineating parallel parking spaces with clear markings, landscaped curb extensions, and appropriate signage, the effective supply of regulated parking can often be increased even as corridor parking is removed. The Terms of Reference specifically call for the Consultant to design structured parking spaces for parking that remains both on- and within 100 metres off-corridor, with landscaped curb extensions between every three to four spaces.

Demand management through pricing represents a powerful tool for reducing the total volume of parking that needs to be accommodated. Yerevan already has the institutional framework for demand-responsive parking pricing through the Parking City Service CJSC and the 2024 tariff reform (Zone A at 300 AMD/hour, Zone B at 200 AMD/hour). By extending or adjusting parking pricing on side streets that receive displaced demand, the municipality can discourage long-stay commuter parking and encourage turnover, freeing spaces for short-term users who genuinely need proximate access. The Mott MacDonald strategy explicitly recommends that parking fees use an hourly rate only, with all period-based fees withdrawn, and that pricing should be set higher than the cost of using public transport on a daily basis.

Park-and-ride at peripheral stations is a strategy often used to intercept car trips before they reach congested central sections, and the 2022 Mott MacDonald strategy recommends it for Yerevan in general terms. It is not carried forward as a measure in this report: the conceptual design for these corridors does not include park-and-ride facilities, it is therefore recorded here as considered and not recommended.

Modal shift facilitated by improved public transport is, in many ways, the most significant displacement mitigation measure of all. The bus priority corridors themselves represent a transformative improvement in public transport quality, offering faster, more reliable, and more comfortable service. International evidence from European BHLS implementations consistently shows that a meaningful proportion of former car users shift to improved public transport when it offers competitive journey times and reliable service. The BRT corridor planning guide published by ITDP notes that in corridors where bus rapid transit has been successfully implemented, a reduction in general traffic demand often materialises, which itself reduces parking pressure. The Yerevan project, by providing dedicated bus lanes, modern electric buses, and upgraded stations, creates the conditions for precisely this type of mode shift.

## Yerevan-Specific Mitigation Framework

Applying the international best practices to the Yerevan context requires careful consideration of the city-specific conditions that shape the feasibility and effectiveness of each measure. Several factors distinguish Yerevan from typical Western European cities and must be accounted for in the mitigation strategy.

First, the existing enforcement capacity, while significantly enhanced by the ANPR camera system operated by Parking City Service, is still developing. The effectiveness of zoned parking controls depends directly on the ability to enforce them. The ANPR control that currently operates provides a strong foundation, but its coverage would need to extend to the side streets that absorb displaced parking. The planned mobile parking application, discussed in the meeting with the PCS Director, would further support enforcement by enabling digital monitoring of parking sessions on newly regulated streets.

Second, the legal and political context around parking in Yerevan is unusually complex. The ongoing judicial challenges to the 2024 tariff reform, the court rulings questioning the empirical basis for parking price increases, and the general political sensitivity around parking charges all constrain the speed and scope with which new parking management measures can be introduced. Any mitigation strategy must be designed to be implementable within the current legal framework or with minimal legislative change, and must be supported by the data-driven evidence that the courts have indicated is necessary.

Third, the urban form of much of Yerevan, particularly the residential areas served by Corridor 2, means that on-street management alone cannot balance parking demand and supply. The microrayon blocks were laid out before mass car ownership and provide little allocated off-street parking, and as car ownership has risen the resulting demand has spread onto whatever surfaces were available, including footways, courtyards and green space.

## Mitigation Measures for Yerevan

As mentioned above, the most important mitigation measure is the bus priority corridor itself. The core logic is that improved BRT-quality public transport reduces dependence on private cars, which in turn reduces the number of trips that require parking. For that reason, the objective should not be to replace every removed parking space on a one-to-one basis. A share of current parking demand is expected to disappear or shift as travel behaviour changes, public transport becomes more attractive, and some users adjust their trip timing, destination, or mode choice. All other parking measures should therefore be understood as complementary to this broader structural effect.

Extend paid zones

The first proposed supporting measure is to extend paid parking zones onto side streets within the corridor influence area. At present, many of the adjacent streets surveyed by the project are outside the paid parking system. Bringing these streets into a red-line or blue-line regime, supported by road markings, signage, hourly pricing, and ANPR-based enforcement, would help absorb displaced demand in a controlled way and prevent uncontrolled spillover. This is particularly relevant where parking is removed from central corridor sections but alternative supply still exists nearby.

Roll out red/blue-line system onto side streets within 100 m of the corridor. Pricing discourages all-day commuter storage, freeing spaces for short-stay users and residents.

About 87% of the kerb is currently unpriced.

Visitor caps

A second measure is the use of visitor caps, meaning maximum parking durations for non-resident vehicles. The logic here is to increase turnover so that the remaining supply serves more users during the day instead of being occupied by the same vehicles for eight or more hours. This is linked to the principle that parking should not operate at full saturation and that some availability should always remain in the system.

Enforce maximum stay durations (e.g. 2 hours) for non-resident vehicles. Increases turnover and prioritises parking availability.

Field-survey verdict: re-scope, do not remove. The data contradicts the premise: about 76% of vehicles already stay one hour or less, all-day parking is only around 6%, and turnover is already 3.5 to 7.5  per space per day. Demote visitor caps to a targeted contingency for genuinely low-turnover spots, lead with pricing, and treat commuters as a BRT mode-shift opportunity rather than a fining target.

Resident permits

Resident permit schemes are a standard instrument for managing kerb space in high-pressure urban areas, particularly in residential neighbourhoods adjacent to major transport corridors. The scheme proposed here rests on two rules that should be stated without qualification. Permits are limited to one per household, issued against a verified home address and vehicle ownership; and permits are charged, at a modest annual fee rather than free of charge. A second vehicle at the same address is not eligible for a resident rate and pays the standard tariff. Both rules matter for the same reason: a free or unlimited permit converts scarce kerb space into a zero-priced entitlement, which is what produces the long-stay, low-turnover pattern the corridor strategy is trying to avoid, and it is very much harder to price a permit later than to price it from the outset. The fee should be set well below the commercial tariff, so that it protects residents against the cost of the new regulation, but not at zero.

The measure is designed to operate in tandem with an expansion of the paid parking zone onto the cross-streets perpendicular to the corridors. Removing on-corridor parking will push displaced demand onto adjacent residential streets, and unless those streets are brought into the paid regime they will absorb the spillover at no cost to non-residents. By pricing the cross-streets and giving residents a low-cost, one-per-household permit, the scheme places the cost of the new regulation on displaced commuters rather than on local households, while still requiring residents to make a positive choice about kerb use. As public transport and non-motorised modal shares rise and car dependency declines, the resident rate should converge on the standard tariff and the distinction can be phased out altogether.

Field-survey verdict: keep, but design it actively, using Tbilisi as a cautionary case rather than a model: Tbilisi currently grants up to two free vehicles per apartment with no paid resident permit - precisely the long-stay, low-turnover pattern to avoid - which is why its new strategy is tightening this to one per apartment.

Organise free parking

In lower-pressure outer zones, a more appropriate response may be to organise free parking rather than price it immediately. This means adding pavement markings, defined bays, and signage to spaces that are currently free but chaotic. The survey results strongly support this approach, since most corridor parking is free and only a small minority of spaces are physically marked or signed. Better organisation alone could improve parking behaviour and make existing supply function more efficiently.

86% of parking spaces on the corridors are unmarked, and 80% are unsigned. They are free, but they are not organised. Physical delineation is expected to influence driver behaviour directly: marked bays discourage the informal positioning that occurs on unmarked kerb, so structuring the existing supply should improve order even before any pricing intervention is introduced.

Delineate and sign currently informal parking - paint bays, add signage, bring order to chaotic areas. No pricing needed yet; just structure. Reduces footpath obstruction and improves pedestrian safety.

Adjust parking charges towards 85% occupancy

Adjusting the existing parking charges to the level at which the kerb actually clears. The commonly adopted occupancy target is 85%, at which roughly one space in seven is free and an arriving driver can find it without cruising. Across whole areas measured occupancy stays below that target, but the pressure is concentrated on particular streets. Of the 203 zones with occupancy data, 34% are at or above the 85% target at their busiest hour and 44 read above 100%. In Kentron, which lies entirely within Zone A, 32 of 53 zones reach the target and 18 read above 100%, although the area as a whole reaches 57% at 12:00. A price that leaves those streets that full is below the market-clearing price there.

This is not a general tariff increase: in Malatia-Sebastia, Shiraz/Hasratyan and Garegin Nzhdeh, where occupancy at the busiest hour runs from 18% to 37%, the same rule gives no reason to raise charges. Where paid zones are extended there, the purpose is to control spillover from the corridor rather than to clear a full kerb, and in the lower-pressure areas organising the free parking comes first.

Setting tariff rates is outside the scope of this assignment, and this report therefore proposes no figure for Zone A or Zone B. Calibrating a rate would in any case require Parking City Service's payment records: hourly payment sessions by zone and time of day, annual and other period-permit sales, and the notices issued for failure to pay. That data was not made available to this study. With it, the charge can be set and adjusted for each tariff zone against the 85% target.

Public acceptance is easier to win where the money visibly returns to the streets that generate it. International practice calls this a parking benefit district: a defined share of the parking revenue raised in an area is spent on visible local improvements in that same area: footway repair, lighting, crossings, planting, reported publicly. The mechanism matters less than the visibility: it is repeatedly what turns residents from opponents of parking charges into supporters of them, because the charge stops reading as extraction and starts reading as a local service.

Withdraw the flat annual parking permit

Among the subscription options offered alongside hourly payment is a flat annual zonal permit, bought once for unlimited use of a parking zone for the year. It works directly against the corridor strategy. A flat annual fee reduces the marginal cost of each additional hour parked to zero, so a permit holder has no financial reason to leave, and the measure that the data identifies as the binding lever on the corridor kerb is precisely turnover. A single permit holder occupying a Zone A space all day displaces the six or seven short-stay visitors that space would otherwise serve.

The measure is to withdraw the flat annual permit and, with it, the weekly and monthly period tariffs, replacing them with hourly occupancy-based pricing and the charged resident permit described above. This is also what the 2022 Mott MacDonald strategy recommends: that parking fees use an hourly rate only, with all period-based fees withdrawn. Withdrawal should be phased, with notice to existing holders and no mid-term cancellation, and paired with the resident permit so that residents who currently rely on the annual permit have a defined alternative.

One qualification is needed on the evidence. Parking City Service has not released permit-uptake or tariff-revenue data, so the share of drivers holding an annual permit, and the revenue at stake in withdrawing it, are not known to this study. That data should be requested from Parking City Service and the withdrawal designed against it, including its revenue effect. The direction of the measure does not depend on the data - a flat annual fee suppresses turnover whatever its uptake.

Remove derelict and long-term abandoned vehicles

A kerb-management framework should include a route for removing vehicles that are not being used at all. A vehicle observed stationary in the same on-street space beyond a stated period is tagged, its registered owner notified through the existing ANPR and police database link, and the vehicle removed to a municipal pound at the owner's cost if it has not moved after a further notice period. Parking City Service can identify candidates from the ANPR record, since a plate detected in the same location across successive days without a payment session is exactly what the system already sees; removal itself sits with the Road Police, which holds the towing power.

Neither this report nor the surveys undertaken establish the scale of the problem, but having such a procedure is recommended nevertheless.

Adding parking capacity in the centre

A further option, raised in review, is to replace part of the removed kerb space by building new parking capacity in the centre, as multi-storey or underground garages. It has real advantages. It would give short-stay visitors and deliveries in the commercial core an alternative close to their destinations, it would take vehicles off footways where the kerb can no longer hold them, and at a single high-demand destination it could answer a specific local shortfall.

The disadvantages are more fundamental. Structured parking is expensive per space, in land as well as construction, and in the centre it competes with higher-value uses of the same sites. New capacity in the core also attracts additional car trips into the area the bus corridors are meant to serve, working against the mode shift that is the primary mitigation. And the field survey shows the central kerb is short-stay dominated and turning over quickly, so the pressure there is better addressed by pricing and managing the existing supply than by adding to it. Adding downtown capacity is therefore not part of the recommended package. Where a new development or a specific destination can demonstrate a shortfall that management cannot close, capacity can be assessed case by case, preferably privately provided and priced at or above the on-street tariff so that it does not undercut the kerb strategy.

Taken together, these measures form a layered mitigation package for Yerevan. They do not seek to preserve the existing parking situation unchanged. Rather, they aim to manage the transition created by bus priority investment in a way that protects access where necessary, improves order and turnover, and supports the broader shift toward more sustainable urban mobility.

Applying that package well matters as much as adopting it. The measures reinforce one another and are meant to be used with judgement rather than uniformly, so their effect depends on how they are sequenced, communicated and adjusted to local conditions. Kerb management is a continuing process rather than a one-off intervention, and the arrangements set out above should be kept under review and refined in the light of experience as the corridors are delivered.

## Phasing and Implementation Considerations

The mitigation of parking displacement should be phased in coordination with the construction and commissioning of the bus priority corridors. Mitigation measures should be implemented in advance of or simultaneously with the removal of corridor parking, not as an afterthought. This principle is well established in European transport planning practice, where cities such as Zurich, Vienna, and Barcelona have demonstrated that early implementation of parking management in the corridor influence area prevents the emergence of problematic overflow parking patterns that are difficult to reverse once established.

A three-phase approach is recommended. In Phase 1 (pre-construction), the municipality should survey and formally delineate all available parking on side streets within 100 metres of the corridor, introduce or extend controlled parking zones in the areas of highest sensitivity, and launch public communication campaigns explaining the changes and the alternatives available. In Phase 2 (during construction), temporary parking arrangements should be provided where feasible, construction management plans should minimise unnecessary disruption to remaining parking, and the improved bus services should begin operating as early as possible to provide a genuine alternative. In Phase 3 (post-commissioning), a monitoring programme should track parking occupancy on side streets, measure any increase in illegal parking, assess the effectiveness of the mitigation measures, and make adaptive adjustments to pricing, time limits, and enforcement intensity based on observed conditions.

This sequencing is a practical requirement rather than a counsel of perfection: removing kerb capacity before the management measures are in place would leave the displaced demand the survey measures without an organised alternative.

Overall responsibility for implementing the package should rest with the Transport Department of Yerevan Municipality, which the Institutional Assessment (Output 28) identifies as the municipality's policy-making and oversight body for urban transport, including parking. Components delivered through the project itself remain with the PIU, as set out in the responsibility matrix of the Institutional and Capacity Building Roadmap (Output 29). Each measure is delivered by the body that holds the relevant powers: Parking City Service, the Police and other relevant agencies.

## Limitations and Residual Risks

It is important to acknowledge that not all displaced parking can be fully accommodated within the surrounding street network. Some net reduction in the total parking supply available within the corridor influence area is an expected and, from a policy perspective, a desirable outcome. The objective of sustainable urban transport is not to preserve the existing level of car accommodation but to create conditions in which car use is less necessary, less convenient relative to alternatives, and less dominant in its claim on public space.

Survey-specific caveats. The field survey observed a daytime window (07:00–24:00) and so under-counts true overnight-residential demand; it covered six representative areas rather than the full corridor; and the absorption figures are gross best-case, counting the total capacity of the nearby streets rather than the share of it actually free at the peak hour, because those streets were not themselves occupancy-surveyed.

# Parking Management Recommendations

## Rationale for Benchmarking in the Yerevan Context

The modernisation of Yerevan's parking system is occurring at a critical juncture in the city's urban evolution. As identified in Chapter 2, the transition from an informal arrangement to a technology-driven municipal service faces significant legal and social headwinds - most notably the active challenges to the 2024 tariff reforms before the Court of Cassation. In this environment, international benchmarking serves as a vital tool for establishing a defensibility layer for municipal policy.

By aligning Yerevan's strategy with the European BHLS framework and the successful reform patterns of post-Soviet capitals - Tbilisi, Tallinn, and Warsaw - the Municipality can support municipal policy with a stronger empirical and comparative basis. International experience demonstrates that successful bus priority corridor implementation requires a fundamental shift: from an assumption of free vehicle storage toward curb management for the highest-value mobility outcomes.

## Precedents and Yerevan Applicability

For the sake of general recommendations to keep in mind, this section evaluates some parking management activities identified in the benchmark cities, synthesising their international performance and specific utility for the Yerevan EMERGE project.

- Controlled Parking Zones (CPZs): Tbilisi's Phase 1 established Zones A and B covering the historic centre and central corridors, managing over 16,000 spaces. Yerevan's red-line system (currently at approx. 2,200 spaces) is recommended to be expanded to include side streets within a 100-metre radius of the new bus corridors to prevent spillover congestion.

- Automated E-Enforcement: Warsaw utilised just three scan cars to monitor 130,000 daily vehicles, achieving a 10% increase in payment compliance with capex recovered within six months. Yerevan achieves the same automation through a fixed network instead: PCS CJSC already operates pole-mounted ANPR cameras, and that asset should be prioritised for strict corridor enforcement and extended to the side streets that take displaced demand.

- Duration-Based Fines: Cities such as Vilnius and Riga employ fines calculated per hour of overstaying. Yerevan's flat 5,000 AMD fine is modest relative to the 160,000 AMD Zone A annual fee. Transitioning to duration-based penalties via the upcoming mobile app would incentivise use of off-street facilities for long-term storage.

- Resident Permits - International Precedents: Comparator cities operate varied permit models - Tallinn applies a modest annual fee (EUR 120) with a two-month residency threshold to prevent fraudulent permit trading, while Bucharest stratifies permit surcharges, with central-district rates up to five times higher than peripheral ones. PCS CJSC currently has no resident permit policy. The scheme proposed for Yerevan (see Section 5.5) adopts a lighter variant tailored to the initial phase of the reform: one charged permit per household at a modest annual fee, no resident rate for a second vehicle, and a linked expansion of the paid zone onto the cross-streets.

- Service-Linkage Revenue Earmarking: Publicly earmarking Zone A revenues for the 249 electric bus fleet would strengthen public communication around the social return generated by parking charges.

## Mitigation Measure Packages by Sensitivity Zones

To reflect the differing parking conditions along the corridors, we have grouped affected areas into three impact or sensitivity zones: high, medium, and lower. Not every corridor segment has the same demand profile, land-use context, or mitigation need, so a single uniform response would not be appropriate. Instead, the mitigation package should be applied selectively according to the sensitivity of each area.

The high-sensitivity zone covers the most pressured sections: Kentron (including streets such as Nalbandyan, Amiryan and Tigran Mets) and Komitas Avenue. These are the areas where parking demand is highest, surrounding activity intensity is greatest, and the removal of corridor parking is most likely to create visible pressure on adjacent streets. For these locations the recommended measures are the strongest ones: extending paid zones, charged one-per-household resident permits and organising free parking, with visitor caps held as a targeted contingency for genuinely low-turnover spots.

The medium-sensitivity zone includes segments such as Arshakunyats, Kievyan and Garegin Nzhdeh, where parking pressure remains substantial but the surrounding context allows somewhat greater flexibility. These areas typically combine residential, commercial and arterial functions, and they may offer more nearby capacity than the central core. Here the preferred package includes extending paid zones, and organising the currently informal parking on the surrounding streets. Compared with the highest-sensitivity areas, the objective is less the strict protection of the centre and more orderly redistribution and operational control.

The lower-sensitivity zone includes outer or more residential segments such as Bagratunyats, Sebastia, Raffi, Nor Nork, Sasna Tsrer, etc. In these areas, traffic intensity is lower, parking demand is generally less concentrated, and the surrounding urban fabric may offer more scope for simple reorganisation measures. For this reason, the response can be lighter and more flexible. The most suitable measure here is organising free parking. In lower-sensitivity areas, the priority is not to intensify regulation for its own sake, but to improve structure and absorb corridor impacts in a practical way.

This zoning approach is important because it turns the mitigation strategy into a place-specific management tool rather than a generic list of measures. The high-sensitivity areas require stronger control and protection mechanisms; the medium-sensitivity areas require managed redistribution; and the lower-sensitivity areas can rely more on reorganisation and local flexibility.

Figure 5 - Summary of the mitigation measures

Figure 6 - Mitigation measures grouped by sensitivity zones

Figure 7 - The map of sensitivity zones throughout the BRT corridors

# Conclusions (and Next Steps)

## Key Conclusions

- Parking loss is substantial and inherent to the design. Design removes 4,843 of the 6,095 on-corridor on-street spaces on Corridors 1 and 2, retaining 1,252. Measured against the impact area, those 4,843 spaces removed are 62% of all on-street parking within it, and 33% of all parking once off-street is included.

- Impacts are highly differentiated by zone. The Kentron CBD and Komitas Avenue face the highest pressure: 32 of Kentron's 53 surveyed zones reach the 85% occupancy target at their busiest hour, and neither area can absorb much of its displaced demand on nearby kerb (16% and 0%). Mid-corridor arterials face moderate pressure; outer residential segments require lighter-touch reorganisation.

- Yerevan has strong enabling infrastructure. ANPR-based enforcement, the red-line registry, PCS CJSC's digital payment ecosystem, and existing municipal authority provide a credible foundation for scaling up a reformed parking regime.

- Mitigation must be layered, not singular. No single measure resolves displacement, that’s why mitigation measures are recommended as packages.

- Enforcement and modal shift underpin demand reduction, but all measures are complementary. Strong enforcement and a credible public transport alternative are the foundation of any successful parking reform, yet they do not operate in isolation. Recommended measures each address a distinct facet of the displacement problem, and their effectiveness depends on being deployed as an integrated package rather than as standalone interventions.

## Further Studies Required

The parking component of this project is scoped to the BRT corridors and their immediate influence area, but parking is inherently a citywide system and cannot be sustainably managed at corridor level alone. The measures proposed here should therefore be understood as the first stage of a broader reform. Consistent with the supply-led, spatially targeted methodology adopted for this assignment, follow-up work should extend the parking supply inventory beyond the corridor buffer to a citywide scale, apply focused demand assessments in selected high-pressure locations rather than corridor-wide observational surveys, and progressively expand the red-line zonal system into neighbourhoods currently outside it. Supporting studies on the legal basis of the resident permit scheme and willingness-to-pay will provide the empirical foundation for calibrating tariffs and guiding the phased citywide roll-out.

# Sources

- "PARKING CITY SERVICE" • ARMENIA (YEREVAN) - SPYUR, https://www.spyur.am/en/companies/parking-city-service/30258/

- Transport - yerevan.am, https://www.yerevan.am/en/transport-department/

- A new procedure of organization of paid car parking will be operated in the capital since July 1 - yerevan.am | Official website, https://www.yerevan.am/en/news/a-new-procedure-of-organization-of-paid-car-parking-will-be-operated-in-the-capital-since-july-1/

- Yerevan to Nationalize Parking Management Firm, https://anca.org/yerevan-to-nationalize-parking-management-firm/

  - The FOICA vs. the "Parking City Service" CJSC - Court cases - Strategic litigation - Freedom of Information Center of Armenia., https://old.foi.am/en/all-cases/item/1315/

- Problems of Machine Processing of Personal Data in Armenia, https://mdi.am/en/archives/1611

- New tariffs for parking on the "red lines" in Yerevan were introduced ..., https://arka.am/en/news/society/new-tariffs-for-parking-on-the-red-lines-in-yerevan-were-introduced-to-relieve-the-center-explanatio/

- Mobile - PanARMENIAN.Net, https://panarmenian.net/m/eng/news/329255

- Court suspends decision raising parking fees in Yerevan - Arminfo, https://arminfo.info/full_news.php?id=92100&lang=3

- ANPR for Smart Parking Management: How It Works - Intozi, https://intozi.io/anpr-smart-parking-management/

- Official website | Municipality - yerevan.am, https://www.yerevan.am/en/staff/

- Parking in Yerevan - Repat Armenia, https://www.repatarmenia.org/repatriate/practical-information/first-steps-after-your-move/parking-in-yerevan

- Sustainability Developements in Armenia | Chambers Expert Focus, https://chambers.com/legal-trends/armenias-quest-for-green-e-mobility

- The Court of Appeal upheld the decision to deem the "red lines ..., https://www.amnews.am/language/en/social-eng-267/

- Vahan Kerobyan proposes to raise parking fees in Yerevan - Finport.am, https://finport.am/full_news.php?id=45957&lang=3

- Court halts parking fee hike in Yerevan - PanARMENIAN.Net - Mobile, https://panarmenian.net/m/eng/news/323133

- New Paid Parking Zones in Central Yerevan: What Drivers Need to ..., https://homycars.com/blog/new-paid-parking-zones-in-yerevan

- Guide to AI ANPR Parking Enforcement Solution in 2025 - Milesight, https://www.milesight.com/security/blog/ai-anpr-and-parking-enforcement

- Enabling smart parking for smart cities using Internet of Things (IoT) and machine learning - PeerJ, https://peerj.com/articles/cs-2544.pdf

- ANPR Car Park System: how it works and benefits - Tattile: Hi-Performance Mobility Solutions & AI software for ANPR, ALPR cameras, https://www.tattile.com/anpr-car-park-system/

- How to Do a Parking Study – MAPC - Metropolitan Area Planning Council, https://www.mapc.org/resource-library/how-to-do-a-parking-study/

- Official website | Electronic services - yerevan.am, https://www.yerevan.am/en/e-service/

- Municipality parking charging me hourly for long term parking : r/armenia - Reddit, https://www.reddit.com/r/armenia/comments/1p5f355/municipality_parking_charging_me_hourly_for_long/

- Odyssey with "Parking City Service" continues: Yerevan Municipality issued new explanations regarding the change in the amount of payment for parking via SMS | Finport.am, https://finport.am/full_news.php?id=30577&lang=3

- Garage Parking Analytics: Key Metrics for Parking Garage Managers - Neighbor Blog, https://www.neighbor.com/storage-blog/key-parking-analytics-for-parking-managers/

- Using Parking Analytics to Optimize Capacity Planning - Parking Guidance Systems, https://parkingguidancesystems.com/using-parking-analytics-to-optimize-capacity-planning/

- Estimating on-street parking occupancy using smart meter data - arXiv.org, https://arxiv.org/html/2106.02270v2

- Model for Determining Parking Demand Using Simulation-Based Pricing - MDPI, https://www.mdpi.com/2076-3417/15/12/6603